- EF-5* – Wall mounted eye/face wash with stainless steel bowl, stainless steel wraparound skirt, ½” U.S. made chrome plated brass stay open ball valve, powder coated cast aluminum flag handle.  Unit shall have (4) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers, filters, and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1721