- EF-24* – Pedestal mounted eye/face wash with stainless steel bowl, (optional ABS plastic bowl “P”) Schedule 40 galvanized pipe and fittings, ½” U.S. made chrome plated brass stay open ball valve, powder coated cast aluminum flag handle and floor flange, stainless steel foot treadle and chain.  Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters, and 3.2-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1760HFC and G1760P-HFC.