- EF-31* – Deck mounted “AutoFlow” swivel eye/face wash less bowl with ½” IPS plug-type valve with Teflon coated O-ring seals and stainless steel enclosure. Supplied with in-line strainer to prevent debris from affecting the valve.  Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters and 3.2-GPM flow control orifices mounted on a chrome plated brass eyewash assembly (Specify spray head configuration).  Activate valve by rotating 90° from stored position.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1779