- EF-32* – Wall mounted, all stainless steel construction, corrosion resistant eye/face wash with stainless steel bowl, ½” U.S. made stay open stainless steel ball valve, stainless steel flag handle and wall bracket.  Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters and 3.2-GPM flow control orifices mounted on a stainless steel head assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1781