- SS-26* - Combination eye/face wash and shower safety station with ABS plastic shower head (optional stainless steel shower head ”SSH”), stainless steel bowl, powder coated cast aluminum flag handle and floor flange, 1 ¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS and ½” IPS U.S. made chrome plated brass stay open ball valves and polished stainless steel pull rod.  Supplied with stainless steel bowl cover which opens when valve is activated, preventing particulates from congregating in bowl.  Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters, and 3.2-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1950BC and G1950BC-SSH