- DH-3* - Wall mounted, hand-held drench hose with ½” IPS U.S. made chrome plated brass stay-open ball valve, insulator handle, powder coated mounting bracket and flag handle, and 12ft. nylon coiled hose.  Unit shall have (2) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers, filters and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5014