- DH-4* - Wall mounted, hand-held drench hose with ½” IPS U.S. made chrome plated brass squeeze valve with renewable stainless steel seat (optional locking clip, “LC”), stainless steel handle with plastic cover, insulator handle, powder coated mounting bracket, and 12ft. nylon coiled hose.  Unit shall have polypropylene ‘FS Plus’ spray head with integral “flip-top” dust cover, filter and 3.2-GPM flow control orifice angled at 45 degrees.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5016 and G5016LC