- DH-5* - Wall mounted, hand-held drench hose with ½” IPS U.S. made chrome plated brass ball valve, insulator handle, powder coated mounting bracket and flag handle, and 12ft. nylon coiled hose.  Unit shall have polypropylene ‘FS Plus’ spray head with integral “flip-top” dust cover, filter and 3.2-GPM flow control orifice.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5018