- DH-12* - Wall mounted, hand held drench hose with ½” IPS U.S. made chrome plated brass squeeze valve with renewable stainless steel seat (optional locking clip, “LC”), stainless steel handle with plastic cover, insulator handle, 8ft. PVC hose, 3/8” IPS swivel inlet, and powder coated wall bracket.  Unit shall have polypropylene ‘FS Plus’ spray head with integral “flip-top” dust cover, filter and 3.2-GPM flow control orifice.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5025 and 5025LC