- EWFR-16* – Pedestal mounted, freeze resistant eye wash with stainless steel bowl, Schedule 40 galvanized pipe and fittings, powder coated cast aluminum flag handle and floor flange, and ¾” IPS stay open bury valve (36” standard bury depth) with ¼” IPS bleed outlet.  Unit shall have (2) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers, filters and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GFR1825