- SSVR-9* - Combination eye wash/shower safety station with chrome plated brass shower head, stainless steel bowl, powder coated cast aluminum flag handle and floor flange, 1 ¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS U.S. made chrome plated brass stay-open ball valve, ½” IPS U.S. made stay-open chrome plated ball valve, and polished stainless steel pull rod.  Unit shall have (2) chrome-plated brass ‘GS Plus’ spray heads with integral stainless steel “flip-top” dust covers, filters, and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.

- Vandal Resistance: All connections shall be secured with setscrews or cement to prevent disassembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2009, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GVR1903